- c

a

p

i

t

- o

- l

- o

7

- ELENCO ALLARMI

- sommario

7.	ELENCO aLLARMI	7-3

PAGINA NON SCRITTA

ELENCO aLLARMI

PAGINA NON SCRITTA

| 000 | Circuit breaker |
| --- | --- |
| Reason | - Overload of protective motor switch - Motor defective |
| Fix | - Check electrical cabinet - Electrical motor check |

| 001 | Temperature drive / inverter |
| --- | --- |
| Reason | - Overload of drives - Filter of fan contaminated - Fan in switch cabinet defective |
| Fix | - Check motor temperature - Change filter - Check if the fan functions |

| 002 | Hydraulic pressure |
| --- | --- |
| Reason | - No pressure build-up in hydraulic system - Pressure loss - leaky pipe work - Pressure switch defective |
| Fix | - Visual inspection for oil loss - Electrical connection of pressure switch |

| 003 | Pneumatic pressure |
| --- | --- |
| Reason | - Pressure loss - leaky pipe work - Pressure switch defective |
| Fix | - Visual inspection of pipe work - Check the pressure switch |

| 004 | PW 210 temperature max |
| --- | --- |
| Reason | - Brake overload drives |
| Fix | - Optimize machine parameters - Reduce feed rate |

| 005 | DA300 pressure |
| --- | --- |
| Reason | - Pressure loss - leaky pipe work - Pressure switch defective |
| Fix | - Change filter - Visual inspection of pipe work - Check the pressure switch |

| 006 | Power supply drives |
| --- | --- |
| Reason | - Supply and recovery unit not operational |
| Fix | - Check supply and recovery unit - Check connection PLC input - Siemens SIMODRIVE 611 ER module (X111 72-73.1) - Indramat Power Drive supply unit HVE (X7 3-4) |

| 007 | Release M07 for Txxx inactive |
| --- | --- |
| Reason | - Tool-specific release for M07 "Coolant, inside" in TOOL.T is inactive. |
| Fix | - If the tool provides M07 "Coolant inside" the function can be enabled via TOOL.T PLC Status #Bit2. PLC status for M07: - %xxxxx0xx = M07 disabled - %xxxxx1xx = M07 enabled |

| 008 | Clamping mode not permitted (Tapping Cycle) |
| --- | --- |
| Reason | - A tapping cycle has been called while the tool axis is in clamping mode. |
| Fix | - Program M- or FN19 function to disable the clamping mode of the tool axis before the cycle call. |

| 009 | Machine guard is closed ! |
| --- | --- |
| Reason | - Guards not opened - Sensors defective |
| Fix | - Open guards - Check sensors |

| 010 | Machine guard is open ! |
| --- | --- |
| Reason | - Guards not closed - Sensors defective |
| Fix | - Close guards - Check sensors |

| 011 | Coolant level low |
| --- | --- |
| Reason | - Coolant level too low - Level switch defective |
| Fix | - Refill coolant - Check the level switch |

| 012 | Lubricating axes |
| --- | --- |
| Reason | - Lubrication cycle of axes not carried out correctly - Pressure loss - leaky pipe work - Pressure switch defective |
| Fix | - Visual inspection for oil loss - Electrical connection of pressure switch |

| 013 | Lubrication oil level low |
| --- | --- |
| Reason | - Oil level of central lubrication system too low - Level switch defective |
| Fix | - Refill oil - Check the level switch |

| 014 | Servo drive release I32 |
| --- | --- |
| Reason | - Axes are ready to power on - Servo drive release I32 is missing |
| Fix | - Check wiring Interface NE2 MC42xx - X42/33 |

| 015 | Servo drive release X150/X151 |
| --- | --- |
| Reason | - Axes are ready to power on - Servo drive release X151/X151 is missing |
| Fix | - Check wiring Interface XX42x - X150/X151 - Check safety contactor combination, both channels should be active. |

| 016 | Servo drive release inverter UM |
| --- | --- |
| Reason | - The axis inverter module (UM) is not READY, no message to CC4xx |
| Fix | - Check PWM cable - Pulse release (spindle or axis) not active - X71/X72 |

| 017 | Central drive |
| --- | --- |
| Reason | - Several axes with central drive were started at the same time |
| Fix | - Change NC program - Each NC block must include only one axis with central drive. |

| 018 | Positioning error TC magazine |
| --- | --- |
| Reason | - Tool magazine blocked or jammed - Tool magazine loaded unevenly - Axes optimization faulty |
| Fix | - Check tool distribution - Optimize axes |

| 019 | Axis motion from end position |
| --- | --- |
| Reason | - One or more of the axes has triggered an over-travel - limit switch resulting in an Emergency stop |
| Fix | - (1) Select Manual operating mode - (2) Close guard - (3) Press "Retract" button - (4) Switch control on - (5) Clear error message Emergency stop (CE key) - (6) Retract with direction keys (see note) |

| 020 | Clamping/unclamping axes |
| --- | --- |
| Reason | - Sensors for clamping/releasing defective - Pressure drop of pneumatic/hydraulic system - [->] Additional note in PLC window: Identifies the axis |
| Fix | - Check sensors and re-adjust them if required - Check compressed-air and hydraulic pressure |

| 021 | Axis not in hirth position |
| --- | --- |
| Reason | - Axis not in Hirth position - Axis movement was interrupted |
| Fix | - Encoder/coupling loose - Compensate reference mismatch via MP960 - Position the axis in manual operating mode on Hirth grid |

| 022 | Nominal position not in hirth |
| --- | --- |
| Reason | - Programmed target position not in Hirth grid |
| Fix | - Correct the NC program |

| 023 | Drives not operational |
| --- | --- |
| Reason | - Axis drive not ready - [->] Additional note in PLC window: Identifies the axis |
| Fix | - Check hardware of axis drive; check diagnosis LED - Switch unit on/off |

| 024 | Power stage temperature max |
| --- | --- |
| Reason | - Overload of servo drive controller - Filter contaminated |
| Fix | - Reduce feed rate - Check / change filter |

| 025 | Drives temperature max |
| --- | --- |
| Reason | - Overload of servo drives |
| Fix | - Check temperature of motors - Reduce feed rate - Switch unit on/off |

| 026 | Axes i2t pre-warning |
| --- | --- |
| Reason | - Overload of servo drives |
| Fix | - Reduce feed rate - Select smaller tools |

| 027 | Axes i2t limitation |
| --- | --- |
| Reason | - Overload of servo drives |
| Fix | - Reduce feed rate - Select smaller tools |

| 028 | Spindle not operational |
| --- | --- |
| Reason | - Overload of spindle - Spindle drive defective - Spindle motor defective |
| Fix | - Reduce feed rate - Check spindle drive - Check spindle motor - Check cable / screwed connection |

| 029 | Spindle nominal rpm |
| --- | --- |
| Reason | - Nominal speed not reached or too low |
| Fix | - Reduce feed rate - Select smaller tools - Check speed with oscilloscope - Optimize machine parameters |

| 030 | Spindle stand still monitor |
| --- | --- |
| Reason | - Offset too high, spindle drifts - Encoder or cable defective |
| Fix | - Compensate offset (analog) - Check encoder (oscilloscope) - Check cable - Optimize machine parameters |

| 031 | Spindle I max |
| --- | --- |
| Reason | - Overload of spindle motor - Spindle motor fan defective/not running |
| Fix | - Reduce cutting values - Check the fan |

| 032 | Spindle T max (sensor Pt100) |
| --- | --- |
| Reason | - Excessive temperature in spindle bearing - Pt 100 temperature sensor defective - Spindle motor fan defective |
| Fix | - Reduce spindle speed - Check Pt 100 temperature sensor - Check spindle fan |

| 033 | Spindle max torque |
| --- | --- |
| Reason | - Maximum permissible torque exceeded |
| Fix | - Reduce feed rate - Increase spindle speed - Select smaller tools - Optimize machine parameters |

| 034 | Touch probe in spindle ! |
| --- | --- |
| Reason | - M03, M04, M05, M13, M14 only permissible if there is no touch probe in the spindle. |
| Fix | - Change the tool (with PLC status %00000000) PLC status for code of tool type - %00000000 = normal tool - %00000001 = touch probe |

| 035 | Touch probe not in spindle ! |
| --- | --- |
| Reason | - Probe function selected but no touch probe in the spindle |
| Fix | - Change the tool (with PLC status %00000000) PLC status for code of tool type - %00000000 = normal tool - %00000001 = touch probe |

| 036 | Spindle 1/2 activation |
| --- | --- |
| Reason | - The switching between spindle S1 and S2 was defective |
| Fix | - Check MP settings (MP4020 bit 5) - Check setting of MP3010/MP13010 |

| 037 | Gear change |
| --- | --- |
| Reason | - The selected gear range cannot be coupled |
| Fix | - Check sensors for the gear ranges - Clear error message (CE key) and retry |

| 038 | Tool not clamped ! |
| --- | --- |
| Reason | - Tool change interrupted - Tool unclamped - Sensors or cables defective |
| Fix | - Press the key "Clamp tool" - Check the sensors |

| 039 | Tool unclamping, please ! |
| --- | --- |
| Reason | - Call for Manual tool change - Tool clamped |
| Fix | - Press the key "Unclamp tool" - Remove the tool |

| 040 | Tool clamping, please ! |
| --- | --- |
| Reason | - Call for Manual tool change - Tool unclamped |
| Fix | - Press the key "Clamp tool" - Insert the tool |

| 041 | TC quit, please ! |
| --- | --- |
| Reason | - Call for Manual tool change - Tool change carried out |
| Fix | - Press the key "Confirm tool change" |

| 042 | Close guard -> TC quit ! |
| --- | --- |
| Reason | - Call for Manual tool change - Tool change carried out |
| Fix | - Close the guard - Press the key "Confirm tool change" |

| 043 | TC end -> NC start |
| --- | --- |
| Reason | - Call for a Manual tool in Auto mode - Tool change carried out |
| Fix | - Press the NC start key to resume program run. |

| 044 | Check pocket table ! |
| --- | --- |
| Reason | - Interruption during tool change - It is possible that the pocket table does not correspond to the loading of the magazine |
| Fix | - Check/correct the pocket table |

| 045 | Correct tool in spindle ? |
| --- | --- |
| Reason | - The tool was unclamped without TOOL CALL |
| Fix | - Check if the tool shown in the status is in the spindle and whether it can be placed in the magazine - NC start to carry out the tool change - NC stop to interrupt the tool change |

| 046 | Spindle is empty ! |
| --- | --- |
| Reason | - There is no tool in the spindle - Sensor defective |
| Fix | - Insert the tool - Check the sensor |

| 047 | Spindle is not empty ! |
| --- | --- |
| Reason | - There is a tool in the spindle although P0 in the pocket table is empty - Sensor defective |
| Fix | - Remove the tool from the spindle - Check the sensor |

| 048 | Tool call T0 necessary ! |
| --- | --- |
| Reason | - Special function in case of tool change of tools with and without magazine pocket - Can be activated via MP |
| Fix | - Change the NC program. First the tool must be removed from the spindle (TOOL CALL 0) before a new tool is inserted. |

| 049 | Tool not in tolerance |
| --- | --- |
| Reason | - Tolerance in tool table exceeded! |
| Fix | - Check the tool |

| 050 | Tool life maximum |
| --- | --- |
| Reason | - Tool life in tool table exceeded! |
| Fix | - Check the tool - Set the current tool life to 0 |

| 051 | Tool change basic position |
| --- | --- |
| Reason | - Tool change interrupted - Error during tool change |
| Fix | - Move the tool change arm/magazine to their 'home' positions by means of the service function (Help)in Manual mode - Test the inputs |

| 052 | Tool change timeout |
| --- | --- |
| Reason | - Timeout during tool change - Interruption of tool change |
| Fix | - Press NC start key to continue the tool change - Press internal stop soft key to stop the tool change |

| 053 | TC magazine guard open ! |
| --- | --- |
| Reason | - Magazine guard open |
| Fix | - Close the tool magazine guard |

| 054 | TC magazine reference |
| --- | --- |
| Reason | - Tool magazine has not been referenced |
| Fix | - NC start to reference the tool magazine |

| 055 | TC magazine timeout |
| --- | --- |
| Reason | - Timeout when positioning the tool magazine - Tool change interrupted |
| Fix | - Press NC start key to continue tool change - Press internal stop soft key to interrupt tool change |

| 056 | TC magazine loading active ! |
| --- | --- |
| Reason | - Loading of the tool magazine selected via soft key - Tool magazine can be traversed and loaded |
| Fix | - Information only |

| 057 | TC magazine pocket not empty |
| --- | --- |
| Reason | - Tool magazine not loaded correctly - Erroneous entries in the pocket table - Sensor defective |
| Fix | - Load the tool magazine correctly - Check the pocket table - Check the sensor |

| 058 | TC magazine pocket is empty |
| --- | --- |
| Reason | - Tool magazine loaded incorrectly - Erroneous entries in the pocket table - Sensor defective |
| Fix | - Load the tool magazine correctly - Check the pocket table - Check the sensor |

| 059 | Wrong tool in spindle |
| --- | --- |
| Reason | - Block scan was stopped - Different tools in status line and spindle |
| Fix | - In the MDI operating mode, insert the tool displayed in the status line |

| 060 | Alternating table timeout |
| --- | --- |
| Reason | - The alternating table cycle started with M61/M62 was interrupted |
| Fix | - Press NC start key to continue the alternating table cycle - Press internal stop soft key to interrupt the alternating table cycle |

| 061 | Enable table 1 ! |
| --- | --- |
| Reason | - M61 starts the NC processing on page 1 (left) but the release key (probe) has not been pressed |
| Fix | - Press the key Alternating table mode on page 1 (left) |

| 062 | Enable table 2 ! |
| --- | --- |
| Reason | - M62 starts the NC processing on page 2 (right) but the release key (probe) has not been pressed |
| Fix | - Press the key Alternating table mode on page 2 (right) |

| 063 | Alternating table not active |
| --- | --- |
| Reason | - M61 or M62 call the alternating table cycle although is has not been activated (soft key or sensor dividing wall) |
| Fix | - Activate the alternating operation by means of soft key or mounted dividing wall |

| 064 | Alternating table X position |
| --- | --- |
| Reason | - When trying to select the alternating table operation, the X axis was in the range of the dividing wall |
| Fix | - Traverse the X axis from the middle in direction table 1/2 |

| 065 | Pallet change basic position |
| --- | --- |
| Reason | - Pallet change interrupted - Error during pallet change |
| Fix | - Move the pallet changer to its' 'home' position by means of the service function (Help) in Manual mode. - Test the inputs |

| 066 | Pallet change timeout |
| --- | --- |
| Reason | - Timeout during pallet change - Interruption of pallet change |
| Fix | - Press NC start key to continue the pallet change - Press internal stop soft key to stop the pallet change |

| 067 | Pallet changer load guard open! |
| --- | --- |
| Reason | - Pallet changer loading guard open |
| Fix | - Close the Pallet changer loading guard |

| 068 | Workpiece not clamped ! |
| --- | --- |
| Reason | - Pallet change interrupted - Workiece unclamped - Sensors or cables defective |
| Fix | - Press the key "Workpiece clamping" - Check the sensors |

| 069 | 3D head timeout |
| --- | --- |
| Reason | - Timeout during 3D head positioning - Interruption of 3D head positioning |
| Fix | - NC start key to continue the 3D head positioning - Internal stop soft key to stop the 3D head positioning - Service menu 3D head in manual mode |

| 070 | 3D head set reference |
| --- | --- |
| Reason | - 3D head reference settings not yet made |
| Fix | - Set reference on service menu 3D head according to the axes position |

| 071 | 3D head G range 1 necessary ! |
| --- | --- |
| Reason | - 3D head positioning is possible only in gear range 1 |
| Fix | - Select spindle rpm on 1st gear range TOOL CALL Sxxx |

| 072 | Insert spindle S2 |
| --- | --- |
| Reason | - M42 has been called to insert spindle S2 |
| Fix | - Insert spindle S2 and press NC start key to continue |

| 073 | Remove spindle S2 |
| --- | --- |
| Reason | - M41 has been called to remove spindle S2 |
| Fix | - Remove spindle S2 and press NC start key to continue |

| 074 | Tool Call from wrong magazine |
| --- | --- |
| Reason | - A Tool has been called, which cannot be changed into the current spindle S1/S2 |
| Fix | - Change NC program |

| 075 | none |
| --- | --- |
| Reason |  |
| Fix |  |

| 076 | PLC 9255 |
| --- | --- |
| Reason | - PLC module 9255 has been caused an error by reading a table - Line: xxx -> Line number of the table the error occured - W1022: xxx -> error recognition |
| Fix | - Please see also module description - Please contact the OEM |

| 077 | Commissioning active |
| --- | --- |
| Reason | - The option "commissioning" has been set in the configuration of machine parameter/MP Config. Several monitoring functions are disabled. - Information only |
| Fix | - When the machine is delivered (factory default setting) the option should be switched off after the commissioning has been completed. |

| 078 | Function not active |
| --- | --- |
| Reason | - Function not executable - Option not active |
| Fix | - Information only |

| 079 | Marker one/zero modified |
| --- | --- |
| Reason | - Error in PLC program |
| Fix | - Please inform OEM |

| 080 | Value not permissible |
| --- | --- |
| Reason | - Function can not be activated because the values are not in the permissible data range. |
| Fix | - Check FN19, M-function and modify values. |

| 081 | Monitoring working area off ! |
| --- | --- |
| Reason | - CAUTION ! - Axis may collide - Monitoring working area has been switched off by soft key or M32. |
| Fix | - Information only |

| 082 | Protected working area ! |
| --- | --- |
| Reason | - Axes are near to a collision point. - In automatic mode axes are stopped, if the distance to go exceeds a collision point. |
| Fix | - Information only |

| 083 | Emergency stop after END PGM |
| --- | --- |
| Reason | - The switching off of the machine after the next END PGM block was activated via user parameters |
| Fix | - Information only |

| 084 | Reference necessary |
| --- | --- |
| Reason | - The reference operating mode is necessary to carry out Function |
| Fix | - Select the operating mode Reference via soft key |

| 085 | Hirth axis initialize |
| --- | --- |
| Reason | - There has been an actual to nominal value transfer for an axis which should keep the nominal value as actual (e.g. hirth axes) - It is no longer guaranteed that the kinematics can be calculated correctly. |
| Fix | - After a positioning the initialisation of the hirth axes will be executed. |

| 086 | Temperature compensation |
| --- | --- |
| Reason | - Error occurred with temperature compensation by means of Pt100 sensors |
| Fix | - Observe the information regarding the error reason in the small PLC window |

| 087 | PLC stack not empty ! |
| --- | --- |
| Reason | - Debug note for PLC programmers - Data was pushed onto the stack and not pulled off. |
| Fix | - Change the PLC program |

| 088 | I'm reading MP's ... |
| --- | --- |
| Reason | - With the first PLC scan or after changes the machine parameters are read in order to adapt the PLC functions - Only after termination of this process can the machine be started |
| Fix | - Information only |

| 089 | M00 programmed stop |
| --- | --- |
| Reason | - Program interruption caused by M00 - Press NC start to continue |
| Fix | - Information only |

| 090 | M01 conditional stop |
| --- | --- |
| Reason | - Program interruption caused by M01 - Press NC start to continue |
| Fix | - Information only |

| 091 | M03/M04 or M19/M20 necessary |
| --- | --- |
| Reason | - To move axes at machining feed rate, one of the mentioned M commands must be active - M31 switches off the function temporarily until the next END PGM - Function can be deselected permanently via MP |
| Fix | - Press NC stop / TNC stop - Correct the NC program |

| 092 | M function not used |
| --- | --- |
| Reason | - Programmed M command is not supported by the PLC program |
| Fix | - Press NC stop / TNC stop - Correct NC program |

| 093 | Feed/Rapid feed pot = 0% ! |
| --- | --- |
| Reason | - Program start with turned off feed/rapid feed rate potentiometer. |
| Fix | - Increase the feed rate on the corresponding potentiometer |

| 094 | Function ok ! |
| --- | --- |
| Reason | - The active step of a HELP file was carried out successfully - All relevant sensors O.K. |
| Fix | - Information only |

| 095 | Check machine parameter ! |
| --- | --- |
| Reason | - The machine parameter setting is not supported by the PLC program |
| Fix | - Change the machine parameter - Observe the information regarding the error reason in the small PLC window |

| 096 | Simulation Maschine aktiv |
| --- | --- |
| Reason | - Simulation of machine function activated by means of control test unit, although due to the I/O assignment a real machine can be presumed |
| Fix | - OEM.CFG: DEFINE CFG_Simulation = "inactive" |

| 097 | Submit/Spawn queue full |
| --- | --- |
| Reason | - More than 8 jobs have been sent to the submit or spawn queue |
| Fix | - Please contact the OEM |

| 098 | TNC programming station |
| --- | --- |
| Reason | - TNC programming station selected (MP7210). - Axes and spindle are not controlled. - Caution ! If PLC is active (MP7210=1), machine functions on simulated axes and spindle positions can be started. |
| Fix | Special functions key combination TExx0 - Keys [Ctrl] + S -> NC start - Keys [Ctrl] + X -> NC stop - Keys [Ctrl] + Cursor up -> axes plus - Keys [Ctrl] + Cursor down -> axes minus - Keys [Alt] + X -> Emergency stop PC keyboard or mouse support possible - Keys [Shift]+[Ctrl]+[Alt]+1 -> vertical soft key 1 ... - Keys [Shift]+[Ctrl]+[Alt]+6 -> vertical soft key 6 |

| 099 | Message in PLC window |
| --- | --- |
| Reason | - Further notes are displayed in the PLC window |
| Fix | - Press CE to clear the display - Press CE to display the next information (if available) |

| 100 | Logbook writing |
| --- | --- |
| Reason | - Data cannot be written to logbook |
| Fix | - Too many submit jobs active - Create data buffer in PLC memory (PLC application) |

| 101 | PLC Module %d(1) - W1022 |
| --- | --- |
| Reason | - PLC module 9xxx was not executed successfully. - (1) -> the first PLC module error during the PLC cycle |
| Fix | - Additional information in line 2 - Please see also module description - Please contact the OEM |

| 102 | PLC Module %d(2) - W1022 |
| --- | --- |
| Reason | - PLC module 9xxx was not executed successfully. - (2) -> the second PLC module error during the PLC cycle |
| Fix | - Additional information in line 2 - Please see also module description - Please contact the OEM |

| 103 | PLC Module %d(3) - W1022 |
| --- | --- |
| Reason | - PLC module 9xxx was not executed successfully. - (3) -> the third PLC module error during the PLC cycle |
| Fix | - Additional information in line 2 - Please see also module description - Please contact the OEM |

| 104 | PLC Module %d(4) - W1022 |
| --- | --- |
| Reason | - PLC module 9xxx was not executed successfully. - (4) -> the fourth PLC module error during the PLC cycle |
| Fix | - Additional information in line 2 - Please see also module description - Please contact the OEM |

| 105 | Check MP data table |
| --- | --- |
| Reason | - Error during reading PLC:\MP\MP_data.tab |
| Fix | - Check values in PLC:\MP\MP_data.tab - If necessary update file from existing backup |

| 106 | Check MP Config Table |
| --- | --- |
| Reason | - Error during reading PLC:\MP\MP_config.tab |
| Fix | - Check values in PLC:\MP\MP_config.tab - If necessary update file from existing backup |

| 107 | RSxxx data transfer PLC |
| --- | --- |
| Reason | - Attempt to synchronize data transmission on serial interface |
| Fix | - Switch on receiver - Check machine parameter MP5xxx of master and slave - Check cable connection |

| 108 | RSxxx buffer overflow |
| --- | --- |
| Reason | - Internal message |
| Fix | - Please contact the OEM |

| 109 | RSxxx transmit error |
| --- | --- |
| Reason | - Error on serial interface RSxxx detected |
| Fix | - Check the cable connection - Press CE key and try to reconnect |

| 110 | RSxxx receive error |
| --- | --- |
| Reason | - Error on serial interface RSxxx detected |
| Fix | - Check the cable connection - Press CE key and try to reconnect |

| 111 | RSxxx not operational |
| --- | --- |
| Reason | - Error on serial interface RSxxx detected |
| Fix | - Check the cable connection - Press CE key and try to reconnect |

| 112 | Table editor blocked |
| --- | --- |
| Reason | - There is a table already open. The MP- or Config file can not be edited. - In operating mode "Traverse reference points" it is not possible to start the table editor. |
| Fix | - Please close the table (e.g. Tool table) and retry. - Please change to mode "Manual operation". |

| 113 | Warning! DCM not yet active |
| --- | --- |
| Reason | - DCM only becomes active after referencing all axes. - To prevent the danger of a collision, the operator must check the current axes positions. |
| Fix | - NC Start key: Movement to a safe position is executed. Subsequently, the reference run of the following axes is started automatically (option). - NC Stop/Axis Direction keys: Automatic reference-run is aborted. The reference run must be performed manually (axis direction keys). |

| 114 | Reference NC start disabled! |
| --- | --- |
| Reason | - Automatic referencing with NC start was aborted. - Automatic referencing cannot be ensured (nonvolatile RAM area not yet initialized). |
| Fix | - The reference run must be performed manually (axis direction keys). |

| 115 | Reference timeout axis |
| --- | --- |
| Reason | - Axis not yet reached safe position |
| Fix | - NC Start key: The traverse to a safe position will be re-started. Subsequently, the reference run of the axes is started automatically (option). - NC Stop/Axis Direction keys: Automatic reference-run is aborted. The reference run must be performed manually (axis direction keys). |

| 116 | Plug/unplug additional axis |
| --- | --- |
| Reason | - To plug or unplug an additional axis the machine must be off. |
| Fix | - Power off the machine ! |

| 117 | Axes compensation changed! |
| --- | --- |
| Reason | - Factor for linear axes compensation X, Y or Z has been changed. |
| Fix | - only message |

| 118 | IPC 61x0 clock monitoring |
| --- | --- |
| Reason | - IPC61x0 interface is faultily (Watchdog) - Operation via IPC has been deactivated |
| Fix | - Try to reconnect by key <Connect> - Check ethernet interface |

| 119 | Operation mode 2 |
| --- | --- |
| Reason | - Information only |
| Fix | - Spindle speed max = S800 - Axes feed max = F2000 |

| 120 | Operation mode 3 |
| --- | --- |
| Reason | - Information only |
| Fix | - Spindle speed max = S5000 - Axes feed max = F5000 |

| 121 | Operation mode 4 |
| --- | --- |
| Reason | - Information only |
| Fix | - Spindle speed max = S5000 - Axes feed max = F5000 |

| 122 | EMERGENCY STOP test required |
| --- | --- |
| Reason | - The EMERGENCY STOP test of the equipment needs to be repeated. - No new NC program can be started. (NC start lock) - A brake test is performed together with the EMERGENCY STOP test. |
| Fix | - After the main switch was switched off, an EMERGENCY STOP test is always run. - The test must be repeated after 168 hours; it starts automatically. Prerequisites: - Doors are closed - No running NC program - Axes and spindle are stopped |

| 123 | Key non-functional |
| --- | --- |
| Reason | In this context the key has no function. |
| Fix |  |

| 124 | EnergyOpt -> Machine switched off |
| --- | --- |
| Reason | - Machine inactive - Axis and Spindle in position - NC Program inactive |
| Fix | - Delite the error with the CE - Switch on the Machine or - Switch off EnergyOpt |

| 125 | EnergyOpt -> AUTOSTART / warmup program active |
| --- | --- |
| Reason | - The machine will be switch on at the specified time automatically by EnergyOpt. - Dependent to the configuration the drives or rather the warmup program will started |
| Fix | - Press the emergancy stop button or - Switch off EnergyOpt |

| 126 | TNC/IP Client 1 |
| --- | --- |
| Reason | - Error while sending data from PLC via TCP/IP sockets. - Timeout. - Connection Error PLC -> Python. - Server not reachable. - Python process crashed. |
| Fix | - Please contact the OEM |

| 127 | TNC/IP Server |
| --- | --- |
| Reason | - Error while sending data from PLC via TCP/IP sockets. - Connection Error PLC -> Python. - Client not reachable. - Python process crashed. |
| Fix | - Please contact the OEM |

| 128 | Error in python mask |
| --- | --- |
| Reason | - A python mask raised an error. - Python process terminated. - Memory allocation error. |
| Fix | - Please contact the OEM |

| 129 | APC door open durning PC |
| --- | --- |
| Reason | - APC door open durning Pallet Change |
| Fix | - Close the APC load door and press key NC start to continue pallet change. - Check the input |

| 130 | M function only in auto mode |
| --- | --- |
| Reason | - The called M function is only in available in automatic mode. |
| Fix | - Call this M function in MDI, Single block or Full sequence |

| 131 | Enable new WP on pallet |
| --- | --- |
| Reason | - M60,M61 or M62 starts the NC processing a pallet change but the READY key has not been pressed |
| Fix | - Press the key enable new workpiece on pallet |

| 132 | APC air leakage |
| --- | --- |
| Reason | - APC air pressure low |
| Fix | - Please check whether chip inside the tape of APC clamp - Check peumatic tube whether leakage - Check whehter clamp position no properly |

| 133 | Pallet clamped oil pressure low |
| --- | --- |
| Reason | - The oil pressure low when pallet clamping. |
| Fix | - Check pressure switch. |

| 178 | Coolant level high |
| --- | --- |
| Reason | - Coolant level to high |
| Fix | - Check coolant level |

| 179 | Paper filter level low |
| --- | --- |
| Reason | - Paper filter level too low |
| Fix | - Refill coolant |

| 180 | TC swarf door error |
| --- | --- |
| Reason | - TC swarf door error |
| Fix | - Check the sensor |

| 181 | Machine guard error ! |
| --- | --- |
| Reason | - The guard is opened when NC progarm is running. |
| Fix | - Check the guard switches. |

| 182 | Cabinet door is opened ! |
| --- | --- |
| Reason | - Cabinet door is opened. |
| Fix | - Close the cabinet door. |

| 183 | CTS pressure high |
| --- | --- |
| Reason | - CTS pressure high |
| Fix | - Checking pipe whether blocked |

| 184 | Operation error |
| --- | --- |
| Reason | - Operation error |
| Fix | - Information only |

| 185 | SM1xx temperature max |
| --- | --- |
| Reason | - SM1xx temperature max |
| Fix | - Note! |

| 186 | CTS coolant pressure low |
| --- | --- |
| Reason | - CTS coolant pressure low |
| Fix | - Check pressure switch |

| 187 | Paper tape filter no paper |
| --- | --- |
| Reason | - Paper tape filter no paper |
| Fix | - Exchange the paper |

| 188 | CTS system error |
| --- | --- |
| Reason | - CTS system error |
| Fix | - Check CTS system |

| 189 | DD motor temperature max |
| --- | --- |
| Reason | - DD motor temperature max |
| Fix | - Cool down the DD motor - Check the temperature sensor |

| 190 | Chiller error |
| --- | --- |
| Reason | - Chiller error |
| Fix | - Check chiller |

| 191 | Warm up required |
| --- | --- |
| Reason | - Spindle off >= 8 hours - needs to warm up about 15 min. - Spindle off >= 1 weeks - needs to warm up about 30 min. - Spindle off >= 1 month - needs to warm up about 65 min. |
| Fix | - Before running the warm up procedure, please load a balanced tool suitable for maximum spindle speed. - Move the Z axis to a safe position. - When ready select MANUAL mode, press and hold the 'SPINDLE CW' key more then 3 seconds, then push 'Cycle Start' to run warm up. |

| 192 | Warm up procedure running |
| --- | --- |
| Reason | - Warm up procedure is running. |
| Fix | - Information only |

| 193 | Warm up interrupted |
| --- | --- |
| Reason | - Warm up is interrupted when warm up procedure is running. |
| Fix | - Information only |

| 194 | Spindle bearing temperature max |
| --- | --- |
| Reason | - Spindle bearing temperature maximum |
| Fix | - Check the spindle chiller - Check the wiring of spindle signal - Cool down the spindle |

| 195 | Spindle shaft compensation |
| --- | --- |
| Reason | - Spindle shaft voltage > +11V - Spindle shaft voltage < -11V |
| Fix | - Check spindle shaft compensation module and wiring |

| 196 | Lubricating spindle |
| --- | --- |
| Reason | - Lubrication cycle of spindle not carried out correctly - Pressure tubings leaky - Pressure switch defective |
| Fix | - Visual inspection for oil loss - Electrical connection of pressure switch |

| 197 | Spindle lubrication - Air pressure low |
| --- | --- |
| Reason | - Pressure loss - leaky pipe work - Pressure switch defective |
| Fix | - Visual inspection of pipe work - Check the pressure switch |

| 198 | Spindle lubrication - Oil level low |
| --- | --- |
| Reason | - Oil level low of spindle lubrication unit |
| Fix | - Refill oil - Check the level switch |

| 199 | Spindle without tool ! |
| --- | --- |
| Reason | - No tool available in the spindle |
| Fix | - Insert the tool - Check the sensor |