DESCRIZIONE DEL PROGRAMMA HELPIO ed HELPIO2.

I programmi HELPIO ed HELPIO2 servono per configurare l'help del Display Input

e Output utente.

Occorre copiare in Ram Disk i due programmi, eseguendo da menu di sistema il

comando Filer Copy COMP:HELPIO*.COD UD:.

Una volta copiati i file in UD:, caricare in memoria di esecuzione il

programma HELPIO2.COD tramite il comando Memory Load HELPIO2.

Editare quindi il programma HELPIO da comando Program Edit e modificarlo

secondo il modello fornito dal programma stesso.

Una volta editato eseguire il comando Program Go HELPIO.

A questo punto, per vedere l'help su un I/O utente, occorre selezionare il

comando di display della variabile (Display Input Ain, Display Input Din,

Display Input Gin, Display Output Ain, Display Output Din, Display Output Gin,

Display Vars Bit e Display Vars Word) e premere il tasto SEL da TP (o CNTRL-END

da WinC4G).

A questo punto il cursore e' posizionato su un bit della variabile e appare una

finestra aggiuntiva che ne descrive il significato.

Per rimuovere la finestra e ritornare nel menu di sistema occorre ripremere il

tasto SEL.

HELPIO and HELPIO2 PROGRAMS DESCRIPTION.

The HELPIO and HELPIO2 programs can be used to configure the Help for

the User INPUT and OUTPUT Display.

The two programs must be copied in UD: with the command

Filer Copy COMP:HELPIO*.COD UD:.

When the files are in UD:, the program  HELPIO2.COD must be loaded

in execution memory with the command Memory Load HELPIO2.

Then the program HELPIO must be edited with Program Edit following

the information included inside the program.

When the editing has been completed, the HELPIO program must be activated

with the command Program Go HELPIO.

Now, to see the Help information related to an I/O point, the  Display

command must be activated (Display Input Ain, Display Input Din,

Display Input Gin, Display Output Ain, Display Output Din, Display Output Gin,

Display Vars Bit e Display Vars Word) and the SEL key from TP

(CTRL-END from WinC4G) must be pressed.

In this situation the cursor is located on a bit of the selected variable

and an additional window shows the meaning of the bit.

To remove the window and go back to the system menu, the SEL key must be

pressed again.