Da: Ramolini, Guido [mailto:Guido.Ramolini@siemens.com] 
Inviato: giovedì 13 settembre 2012 16:33
A: carlo.spaziani@rem-motori.it
Cc: Gasparotto, Massimiliano (EXT); Boccazzi, Lorenzo
Oggetto: R: COMUNICAZIONE

| Company REM attn. Mr Spaziani |  | Name | Ramolini, Guido |
| --- | --- | --- | --- |
|  |  | Department | SWE RC-IT I DT LD LO&SYS |
|  |  | Telephone | +39 (02) 243-64382 |
|  |  | Fax | +39 (02) 243-62216 |
|  |  | E-Mail | guido.ramolini@siemens.com |
|  |  | Our reference | AGT-400613-QUOTATION NOT RELEASED! |
|  |  | Date | 13/09/2012 |

Commercial information your ref SPARE motor

Dear Mrs. / Mr.  ,

We thank you for your enquiry and have pleasure in putting forward our proposals for the above project:

| Item: | 01001 |  |
| --- | --- | --- |
|  | Quantity/ Type: | 1 Medium voltage motor |
|  | Order code: | 1LA4 314-6AN09-Z |
|  | Accessories: | A65+B15+G50+K20+K84+M13+Y55 |
|  | Description: | -A65: 6 embedded resistance thermometers PT 100 without lightning arresters. -B15: Retrofit -G50: Shock pulse measuring (SPM), measuring nipple at DE and NDE -K20: Heavy duty rolling contact bearings -K84: System connection on right-hand side at drive end, cable from NDE -M13: Anti-condensation heating 220-240V (min. 200V, max. 264V) -Y55: Abnormal cylindrical shaft extension on drive end |
|  | Others per unit: | -Mounting IM V5, NDE brg. 72.. (V1 bearings used if calculated) |
|  | Tests: |  |
|  | Others per item: |  |
|  | Techn. Data: | 235 kW 3.3 kV 50 Hz 986 rpm; IM V1; IP55; IC 411; ohne |
|  | Delivery time: | 100 Working day(s) |
|  | Comment: | Replacement for 1LA1 352-6HE09-Z, serial no. 094153...155 (works no. 61599/1, 1991) Mounting IM V5, no canopy. Reinforced DE bearing NU220E. The NDE bearing is the thrust bearing! Shaft end D95x170 as on the original motor. Main terminal box at DE (right side) with cable entry from NDE. Please confirm the terminal box position with the customer! Please note that the replacement motor is in shaft height 315, not in shaft height 355 as the original motor. An adaptor frame for IM V5 motors is not possible and not purposeful. The customer must modify the wall fixation and support bracket to accomodate the replacement motor. |
|  | ECCN: | N |
|  | AL: | N |

Item net price: K = 44.000EUR

Delivery Time:

The delivery time ex works stated in the items is valid if your order is clarified in all technical and commercial details.

Validity:

This quotation is valid until 11.12.2012.

Commercial Condition:

The prices quoted are on the basis DDP Incoterms in the latest valid issue including our standard documentation. Additional or special documentation could be the subject of additional costs.

Payment shall be made as follows:

1/3     Advance payment
  1/3     At half delivery time
  1/3     At time of delivery

In all other respects the "General Conditions for Supplies of Siemens, Automation and Drives for Customers with a Seat or registered Office outside of Germany", status 01 January 2002 shall apply.